X-FAX PRINTER DRIVER FORMAT

---------------------------

File: C:\X-FAX\EPSON.DRV

��������������������������������������������������������������������������������

Offset        Hex                            �� Begin of the sequences

000000        00 00 B4 00  03 64 B4 00  03 64�1B 2A  27 FE FF 00

000010        00 00 00 00  00 00 00 00  00 00 1B 2A  27 FE FF 00

000020        00 00 00 00  00 00 00 00  00 00 1B 30  10 0A FF 00

000030        00 00 00 00  00 00 00 00  00 00 1B 40  FF 00 00 00

000040        00 00 00 00  00 00 00 00  00 00 0C 1B  40 FF 00 00

000050        00 00 00 00  00 00 00 00  00 00

��������������������������������������������������������������������������������

OFFSET  LENGTH (BYTES)          DESCRIPTION

0           2                 Printing mode. 00 = Epson standard

01 = HP PCL standard

2           2                 DPI in HIGH QUALITY mode

4           1                 Bytes per column HIGH QUALITY

If you use the PCL standard, this must always

be 1!

5           1                 Scaling-adjustment X-axis (100 is standard)

The higher the value, the smaller the picture.

( i.e. 101 would be smaller than 100 )

6           2                 DPI in LOW QUALITY mode

8           1                 Bytes per column LOW QUALITY

If you use the PCL standard, this must always

be 1!

9           1                 Scaling-adjustment y-axis (100 is standard)

The higher the value, the smaller the picture.

( i.e. 101 would be smaller than 100 )

The following bytes are the five initstrings X-FAX needs to communicate

with your printer.

10          16                High Quality Init (Set printing mode)

26          16                Low Quality Init (Set printing mode)

42          16                Set line spacing (mostly 1/8") + LF

58          16                Printer reset.

74          16                Form feed (FF)

Note that you may use the following macros for the init strings:

FD          Bytes per line (PCL mode only)

FE          Bytes per line (Epson mode only)

FF          End initstring (You MUST terminate every init-string with this!)