V.32 terbo

19,200 bit/s Full Duplex Modem

for the

General Switched Telephone Network

Compatible with V.32 bis

Document Number:  3800-A2-GK21-00

V.32 terbo

19,200 bits/s Full Duplex Modem for the General Switched Telephone Network

Compatible with V.32 bis

Document Number:  3800-A2-GK21-00

May 1993

COPYRIGHT ã 1993 AT&T Paradyne Corporation

AT&T Paradyne Corporation grants permission to duplicate this document for non-commercial purposes only.

AT&T Paradyne Corporation, 8545 126th Ave. N., P.O. Box 2826, Largo, Florida 34649-2826.

1.0  Introduction

This specification identifies new data signaling rates of 16,800 bit/s and 19,200 bit/s for use with V.32 bis modems.  Pertinent sections of the V.32 bis recommendation are modified to reflect the new data signaling rates.  These are section 2.3 on Coding and section 5.3 on the Rate signal.  All other characteristics of the modem are identical to V.32 bis.

2.3  Coding

2.3.1  Signal element coding for 14,400, 16,800, and 19,200 bit/s

At 14,400, 16,800, and 19,200 bit/s the scrambled data stream is divided into groups of 6, 7, or 8 consecutive data bits, respectively.  The first two bits in time Q1 and Q2 are differentially encoded and trellis encoded following Recommendation V.32 bis to generate 3 trellis encoded bits Y0, Y1, and Y2.  These trellis bits and all remaining information bits, Q3 through Q8, are mapped into the coordinates of the signal elements as identified in the signal space diagrams.  At 14,400 bit/s the signal space diagram is as shown in Recommendation V.32 bis.

At the new data rates of 16,800 and 19,200 bit/s the signal space diagrams are shown in Figure 1/V.32 terbo and Figure 2/V.32 terbo respectively.  Each signal element is identified in hexadecimal format. Y0 is the least significant bit in the least significant hexadecimal digit.  Q4 is the least significant bit in the second hexadecimal digit.  At 19,200 bit/s, Q8 is the least significant bit in the third hexadecimal digit.  At 16,800 bit/s the signal space diagram is a subset of the 19,200 bit/s signal space diagram.

2.3.1.1  Nonlinear Encoding

At the new data rates of 16,800 and 19,200 bit/s the signal space diagrams are modified by nonlinear encoding.  Nonlinear encoding is enabled only during data transmission at these rates and not during other sequence states.  Nonlinear encoding is disabled during start-up and retrain.

Each signal space point is nonlinear encoded by

X = Fx  and  Y = Fy

where the nonlinear encoding projection function is

F  = K(16384 + 2731P + 137P2+ 3P3)/16384

and P = C(x2 + y2) .  Note that C = g/R2 is a constant where the nonlinear encoding gain is g = 0.8 and R2 is the square of the radius of the signal space at the data signaling rate of 19,200 bit/s.  The transmitter gain correction factor is

K = 1 - 1697g/16384.

All data rates require scaling of the signal space diagrams to achieve equal transmit power.  It is recommended to perform nonlinear encoding after this power adjustment.  In this case the radius of the signal space at 16,800 and 19,200 bit/s are approximately equal.  The radius at 19,200 bit/s can be used to calculate C at both data rates so that identical equations are used.

2.3.1.2  Nonlinear Decoding

At the new data rates of 16,800 and 19,200 bit/s the signal space diagrams are modified by nonlinear decoding at the output of the equalizer in the receiver.  Nonlinear decoding is enabled only during data reception at these rates and not during other sequence states.  Nonlinear decoding is disabled during start-up and retrain.

The reciprocal nonlinear decoding operation is performed at the equalizer output

x = RX and y = RY

using the reciprocal function

R = (1/K)(16384 + 2731P + 1229P2 +  731P3  + 264P4)/16384

where P = -C(X2 + Y2)/K2. The constants C and K are defined in 2.3.1.1 above.  K is the transmitter gain correction factor.  The recovered points x, y are decoded using conventional techniques.

Note that in V.32 terbo the nonlinear decoder is only active during data transmission at the data rates of 16,800 and 19,200 bit/s.  The nonlinear decoder is not active during training.  Training is completed before the rate  sequence is detected.  For this reason the transmitter gain correction factor K is incorporated in the calculation of the nonlinear decoder.  The equalizer would remove this constant gain factor if the nonlinear decoder were active during training.

5.3 Rate Signal

The rate signal is defined in Recommendation V.32 bis.  Bits 13 and 14 are modified to identify the new data signaling rates:

B13	1 denotes that operation at 16,800 bit/s rate is enabled

B14	1 denotes that operation at 19,200 bit/s rate is enabled

Hexadecimal numbers refer to Q7Q6Q5Q4 and Q3Y2Y1Y0

Figure 1/V.32 terbo

Signal space diagram and mapping for 16,800 bits per second

Hexadecimal numbers refer to Q8, Q7Q6Q5Q4 and Q3Y2Y1Y0

Figure 2/V.32 terbo

Signal space diagram and mapping for 19,200 bits per second