... HISTORY.DOC .......................... TMi0SDGL(tm) Companion Document ...

Document no.: N-0014.94.95M01.08

TMi0SDGL(tm) Version 1.16 - HISTORY.DOC

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

History of changes.

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

TMi0SDGL(tm) version 1.10d was the first version publicly released.

Differences between version 1.10d and version 1.11:

- changed 80287XL determination logic: now this conclusion is made if CPU

is 80286 and FPU tests as 80387;

- added IIT (Integrated Information Technology, Inc.) xC87 math coprocessors

detection;

- fixed bug in 386 stepping detection routine.

Differences between versions 1.11 and 1.11b:

- fixed bug that caused FPU_Type routine to hang on 486s: they don't handle

IIT bank switching instructions at all, stopping CPU.

Differences between versions 1.11b and 1.12:

- distinguish between Cyrix 486SLC and 486DLC made reliable;

- code distinguishing between 386sx and 386dx that used bug in dx chips

deleted (it was commented out in previous versions) - current method is

absolutely reliable;

- code slightly optimized to reduce size.

Differences between versions 1.12 and 1.12b:

- some slight modifications made to CPUSPEED.ASM module to prevent invalid

computation of clock frequency for unknown CPUs.

Differences between versions 1.12b and 1.13:

- added 386sl detection routine, but wasn't checked yet: at least it doesn't

hang the PC :);

- distinguishing between i486sx and i486dx finally made reliable;

- code slightly optimized;

- CPUSPEED.ASM enhanced and recommented to reflect new CPU type addition.

Differences between versions 1.13 and 1.14:

- reduced source file size of CPUSPEED.ASM;

- Test_Buffer routine made reentrant ( at last! :);

- added newer 386 and 486 recognition ( ID flag of EFLAGS is flippable and

CPUID instruction works on new 386s and 486s so I took precaution of

this situation);

- described method of identifying AMD's 40MHz 386s and a little bit tricky

method of identifying i486DX2/66 chips;

- added method of identifying Cyrix M1 (586) chip - was not checked yet;

- 386SL detection code commented out, but still present - maybe it works on

some chipsets...

Differences between versions 1.14 and 1.14b:

- added hi-level interfaces and sample programs,

Turbo Pascal 7.0, Turbo C 2.0 and Borland C++ 3.1 were used to compile these

examples, they appeared to work o.k.;

- added i487sx detection;

- corrected Cyrix chips id's - there are also SX and DX chips;

- added Known Incompatibilities list;

- wow! this thingie's gettin' really BIG, ain't it..?

Differences between versions 1.14b and 1.14c:

Lots o'bugs fixed in this version:

- fixed bug in hi-level routines that caused Intel 386/33 chips to be

identified as AMD ones;

- finally got rid of DOS session crashes under Windoze in assembly-language

version on 386dx-based systems; also, the code made Windoze-aware in

time-critical sections so it won't bother me (and you) anymore;

- added makeX.bat batches to simplify compile process. Of course paths and

parameters should be adjusted in 'em;

- finally fixed code for new 386+ chips which handles CPUID instruction -

this command appeared to destroy some registers I used to hold temp data.

Too bad I haven't Pentium data sheet :( - this bug could never appear...

- added new module P5INFO.ASM and two sample programs in Pascal and C demon-

strating it's usage - Pentium (and predecessors with cpuid enabled) feature

info lister. This is rough approach - when I have a P5 data sheet I'll add

more info;

- fixed OS/2 DOS Session crash problem on 486 systems - that ugly OS doesn't

allow to flip NE bit of CR0 hanging up the PC;

- this thing became really BIG now - 36K in ZIP. I think it's time to stop.

But I'll still develop it if you, users, will tell me I should.

Differences between versions 1.14c and 1.14d:

- I took some time and finally adjusted CPU timings so that CPUSpeed routine

produce more precise results but it still may give inadequate values on

certain CPUs from different vendors - there's so many of them;

- fixed obvious bug in P5Info - Make string is returned by CPU itself in

EBX,EDX,ECX registers, so I fixed it;

- made an approach to recognize UMC's U5 chips in hi-level routines;

- slightly modified hi-level interfaces to adapt them to new approach;

- optimized OS/2 detection routine;

- added Note 7 in dox - PLEASE READ IT BEFORE SENDING ME BUG-REPORTS!;

- stripped comments from CPU_HL.ASM - this is just a port from CPU_TYPE.ASH

which contain 'em all;

- added integer-only CPU speed calculation routines (got rid of fp), although

you'll lose precision using them;

- added DESQview calls Begin/End critical section in CPUSPEED.ASM;

- tested under Quarterdeck DESQview 2.63 - caused no probs on my Am386DX-40

system, MHz computed correctly.

Differences between versions 1.14d and 1.14e:

- Fixed an obnoxious bug in CPU_HL.ASM that caused P5 misidentification;

- fixed reporting Cyrix chips - only SLC is now distinguished from other

Cyrix's 486s;

- again fixed P5Info report - feature flags were shifted somehow;

- Note 7 moved to separate file README.1ST, and you SHOULD READ IT.

Differences between versions 1.14e and 1.14f:

- SMM test added, batches adjusted to reflect this addition;

- added one more P5 feature flag description to P5Info programs.

Differences between versions 1.14f and 1.14g:

- rewritten OS/2 detection routine, several bugs under OS/2 evaporated;

- added 486 SX/DX/DX2/DX4/SX2/SL/Overdrive distinguishing for newer CPU models

with CPUID support (thnx 2 Janis Smits once again for model codes), but

this new code is not tested yet!!! It shouldn't cause any trouble but it may

misidentify some 486s...

- some other bugs are being chased right now :-E~ ...

Differences between versions 1.14g and 1.14h:

Damn it! Previous version was full of bugs :(

- finally made the code able to run in protected mode. Now you can really

uncomment those commands in makep.bat that create protected mode .exe;

- found that old 486SX/DX detection method doesn't work under Borland's DPMI

host. I gotta find other way to determine this...

- fixed bug that prevented new 486SX/DX/DX2/etc. detection code from

execution, also avoided FPU checking for new CPUs with FPUonChip flag set;

- calling convention changed to pascal everywhere (though there should not

be any probs in upgrading your C/C++ applications for the rest of library

was not changed);

- added MAKEFILE for those who prefer using MAKE to build projects;

- some other minor fixes made.

Differences between versions 1.14h and 1.15:

- added new code detecting 386 and 486 model without using CPUID instruction

and sample program demonstrating its usage;

- makefile updated to reflect this addition;

- added FPU emulator detection;

- added P24D detection code;

- Cyrix misidentification under Windows fixed;

- C code rearranged for compatibility with Microsoft C[++].

Differences between versions 1.15 and 1.16BETA:

- some spelling mistakes fixed in several files;

- MAKEC.BAT and MLIB.BAT modified to simplify paths setting. Now environment

variables INCLUDE and LIB used to determine corresponding paths;

- added Cyrix CPU models distinguishing code by Sergei Frolov;

- added code that should distinguish IBM's 386SLC, 486SLC and 486SLC2 chips;

- assembler version of code rewritten to allow CPU speed report;

- fixed some bugs appearing on 8086 and 80186 (I recently found machines with

these ancient CPUs installed and found that TMi0SDGL hangs on them, so I

took a look inside and fixed it);

- changed P24D detection code, added P6 detection code

(!!! P6 speed timings needs to be adjusted);

- fixed obvious bug in SMM.ASM that caused GPF on systems without SMM support;

- fixed 386 detection routine in SMM detection module which failed on 8086.

Differences between versions 1.16BETA and 1.16BETA_2:

- fixed MAKEP.BAT and YESNO.COM;

- added Texas Instruments Ti486SXL detection;

- tried to fix hangup problem on 8086s (changed all FINITs to FNINITs);

- changed U5 chips detection code - now it should detect all steps of U5;

- CPUs with SMM support are now claimed as "SL Enhanced";

- the License Agreement slightly edited;

- some spelling corrections made in various places.

Differences between version 1.16BETA_2 and 1.16:

(three ALPHA test stages and pre-release version

were released as executables only)

- fixed integer printing routine in CPU_TYPE.ASH (it worked incorrect with

integers >= 100);

- P5INFO programs slightly enhanced to give a try for CPUs with no ID flag;

- Pascal version of CPUSpeed and intCPUSpeed routines rewritten for maximum

performance and precision;

- AMD 386 detection code removed - it was not worthy;

- IBM 386SLC/486SLC/486SLC2 detection code rewritten, 486BL3 (Blue Lightning)

detection code added;

- QEMM-386 detection code commented out everywhere and IBM's SLC chips

detection left to work only in real mode - QEMM appeared not to pass GPF

exception to DOS real mode interrupt handler which is not acceptable for

current detection method: RDMSR with invalid MSR index causes GPF and we

should be able to handle it. The code still present though - maybe someone

can make use of it anyway *smile*;

- SMM support detection added to assembler version (activated if

__PRINT_STEP__ NOT defined);

- finally made it work on 8086 *relaxed smile*;

- added some IFs for TASM 4.0 (it supports P5 instructions);

- finally made UMC U5 detection work in assembler version;

- finally got rid of incorrect 487sx detection on new 486s with CPUID and

internal FPU;

- fixed incorrect FPU detection on Ti486SXL (stated "Internal" while there's

no FPU on chip);

- numerous FPU types detection added (ULSI, C&T, etc.);

- RapidCAD is now history, so I decided not to include its detection in

TMi0SDGL though now I know how to do it;

- fixed FPU detection routine (labels got messed up);

- added NexGen Nx586 and its companion FPU Nx587(not yet available) detection;

- fixed Internal FPU detection, some obsolete code eliminated;

- fixed bug in print_CPU routine (assembler version) which caused i486 model

to be reported incorrectly;

- tried to fix OS/2 trap in FPU detection routine;

- IIT FPUs detection method changed;

- Pascal and C code slightly optimized;

- now if 486sx along with FPU is detected, this leads to conclusion that this

is 486DX or 487SX with internal FPU;

- added P5/P54 distinguishing;

- added new AMD Am486DX2, DX2+, DX4, DX4+ CPUs detection;

- documented Windows 95 crash problem on SMM support test;

- iP24T given its real name everywhere.

<< end of HISTORY.DOC >>